From Class #1

7 OSI Layers

7- Application: Network process to application

- Email, telnet, www

6- Presentation: Data representation

- Format of data

5- Session: Interhost communication

- Client/server

- Establishes/terminate sessions btw applications

4- Transport: End to end connections

- TCP vs UDP

- Virtual Circuits

3- Network: Data Delivery

-  IP address, TCP, routers

- best path

- logical addressing

2- Data Link: Access to media
 	- MAC/LLC (logic link control), Bridges, Switches

1- Physical: Binary Transmission

-  cables, hubs repeaters

Data Encapsulation:

- From OSI layer 7 -> 1

Data Decapsulation:

- From OSI layer 1 -> 7

Peer to Peer Communication:

- Application ->  	Data Message

- Presentation ->	Data Message

- Session ->		Data Message

- Transport -> Segments

- Network -> Packet

- DataLink -> Frames

- Physical -> Bits

Internet protocol Characteristics

- Operate at layer 3 (network)

- Connectionless

- Independent packets

- Hierarchical addressing (class A,B,C,D,E)

- Best effort delivery

- No data recovery features

PDU – Packet Data Unit

IP Address:

- 32 bit / 4 octet

- Classes

  - A:   Network.Host.Host.Host

    - 0xxxxxxx.host.host.host

    - (1-126).xxx.xxx.xxx

  - B:	Network.Network.Host.Host

    - 10xxxxxx.network.host.host

    - (128-191).xxx.xxx.xxx

  - C:	Network.Network.Network.Host

    - 110xxxxx.network.network.host

    - (192-123).xxx.xxx.xxx

Special restricted/Reserved IP address:

- 127.xxxx.xxx.xxx 		Reserved for loopback

- Network.00000000		Network address

  - If subnetted, the first host address

- Network.11111111		Directed broadcast address

  - If subnetted, the last host address

Public IP Address (Visible in the WAN must be unique)

Class A: 	1.0.0.0 -> 9.255.255.255

11.0.0.0 -> 126.255.255.255

Class B:		128.0.0.0 -> 172.15.255.255

172.32.0.0 -> 191.255.255.255

Class C: 	192.0.0.0 -> 192.167.255.255

192.169.0.0 -> 223.255.255.255

Private IP address (Visible only inside the organization)

Class A: 	10.0.0.0 -> 10.255.255.255

Class B:		172.16.0.0 -> 172.31.255.255

Class C: 	192.168.0.0 -> 192.168.255.255

DHCP -> clients renew lease when it is 50% done

DNS - > translate human-readable name to IP address

TCP Vs UDP

| TCP (Reliable) | UDP (Best – Effort) |
| --- | --- |
| - Connection Oriented - Sequencing of packets - Email, file sharing, downloading - Full-Duplex mode operation - error checking - acknowledgement of receipt - Data-recovery features (flow control) | - Connectionless - No sequencing - voice / video streaming - best- effort delivery - has no data-recovery features - 16-bit source port - 16-bit UDP checksum |